#### NSX Architecture — Low Level Design

The following section documents the specific NSX architecture for the American Express VCF 9 on-premises deployment across two availability zones. This design implements the Dual Availability Zone Design based on NSX Edge Node HA model, with dedicated NSX Edge Clusters per rack and NSX Federation providing centralised policy management across both Local Managers.

##### Architecture Overview

The NSX architecture spans AZ1 (Rack 1) and AZ2 (Rack 2) within a single data centre region. Key architectural characteristics of this deployment are as follows:

- NSX Federation with a three-node Global Manager cluster provides centralised security policy and segment management across both NSX domains.

- Each availability zone hosts an independent three-node NSX Local Manager cluster with a Virtual IP (VIP) for API and UI access.

- Dedicated NSX Edge Clusters per availability zone host the NSX Edge nodes. Edge nodes cannot move between availability zones.

- A single Tier-0 gateway operates in Active/Standby mode with the Active Service Router hosted on AZ1 edge nodes and the Standby Service Router on AZ2.

- A single Overlay Transport Zone is shared across all clusters within each NSX domain, ensuring all VPCs and overlay segments are available to all ESXi hosts and Edge nodes.

- Fleet-level management components (VCF Automation, VCF Operations, VOL, VON, Identity Broker) are deployed on an NSX overlay segment via the T1 gateway, providing addressing portability across both availability zones.

- Distributed Firewall (DFW) is enforced on all ESXi host vNICs across both availability zones, with policy pushed by the NSX Local Managers and rules defined in the Firewall Rules Workbook.

##### NSX Design Decisions

| Decision ID | Design area | Decision / selection | Implication |
| --- | --- | --- | --- |
| NSX Management Plane |  |  |  |
| VCF-NSX-LM-REQD-CFG-005 | NSX Manager deployment | Three-node HA cluster per AZ with internal VIP. All nodes on VM Management Network (VLAN 2600). | Anti-affinity rule required — all 3 nodes on separate ESXi hosts. Minimum 4 physical hosts needed per AZ. |
| VCF-NSX-LM-RCMD-CFG-002 | NSX Manager anti-affinity | VM-VM anti-affinity rule applied via vSphere DRS to all NSX Manager appliances. | Cluster continues operating on surviving hosts in the event of a single ESXi host failure. |
| VCF-NSX-LM-RCMD-CFG-002 (Stretched) | NSX Manager AZ placement | All NSX Manager appliances added to the VM group for AZ1 (primary availability zone). | Ensures NSX Manager nodes are powered on in AZ1 by default. No impact to AZ2 availability. |
| NSX Federation | NSX Federation model | Global Manager (3-node) pushes global policy to Local Managers via TCP 443 (API sync) and TCP 1236 (async replication). | TCP 1236 MUST be whitelisted GM ↔ LM in both directions. Without it, global policy never reaches Local Managers. |
| NSX Edge Design |  |  |  |
| VCF-NSX-EDGE-REQD-COMDAZ-01 | Edge TEP MTU | Edge node TEP MTU aligned to vDS MTU — 9000 bytes throughout. | Requires manual configuration of TEP MTU value in NSX Global Fabric settings. |
| DD-MGT-009 | NSX Edge cluster model | Dual AZ — dedicated vSphere cluster per rack. EN1/EN2 in Rack 1, EN3/EN4 in Rack 2. | Edge nodes cannot move between racks. BGP peering is to local ToR switches only — never cross-rack. |
| DD-MGT-010 | T0 Gateway model | Active/Standby. AZ1 hosts the Active Service Router, AZ2 hosts the Standby Service Router. | Inter-SR BGP (TCP 179) and BFD multi-hop (UDP 4784) required cross-AZ for T0 routing convergence. |
| DD-PHY-006 | BGP peering design | eBGP from each edge node to its local ToR switch only. VLAN 2606/2607 for Rack 1, VLAN 2608/2609 for Rack 2. | BFD required on all uplinks: UDP 3784 (control), 3785 (echo), 4784 (multi-hop). All three must be whitelisted. |
| Transport Zones & TEP |  |  |  |
| VCF-VNI-REQD-CFG-003 | Overlay Transport Zone | Single overlay transport zone for all clusters in the NSX domain. Shared by management and workload domain clusters. | Ensures all overlay segments and VPCs are available to all ESXi hosts. Sub-transport-node profile created per AZ. |
| VCF-VNI-SC-REQD-CFG-001 | Host TEP uplink profile | Separate uplink profile per AZ with unique Host TEP VLAN per rack. | Enables different VLAN IDs for Host TEP in each AZ. Additional Host TEP VLAN required in AZ2. |
| DD-VLN-008 | Edge TEP (VLAN 2603) | Intra-AZ HA and GENEVE encapsulation. AZ1 subnet and AZ2 subnet assigned. Cannot be NATted. | GENEVE UDP 6081 must be open between Host TEP and Edge TEP subnets in both directions. MTU 9000 end-to-end. |
| DD-VLN-009 | Remote TEP — RTEP (VLAN 2605) | VLAN 2605 stretched cross-rack. Used for cross-AZ GENEVE overlay traffic. Cannot be NATted. | Physical fabric must carry VLAN 2605 between Rack 1 and Rack 2. AZ1 and AZ2 RTEP subnets must be mutually routable. |
| Security & Segmentation |  |  |  |
| DD-SEC-007 | Gateway Firewall | Enforced on NSX Edge nodes. Controls all north-south traffic entering and leaving the overlay. | Rules defined in Firewall Rules Workbook (AES-220). Applied at edge node level for all uplink traffic. |
| DD-SEC-007 (DFW) | Distributed Firewall | Micro-segmentation enforced on all ESXi host vNICs across both AZs. Policy pushed by Local Managers. | Applies to Fleet VMs, Workload VMs, and Management VMs. Rules sourced from Firewall Rules Workbook. |
| Fleet Overlay Segment |  |  |  |
| DD-MGT-011 | Fleet component placement | All fleet VMs deployed on NSX overlay segment (T0/T1 gateway). Segment spans both AZs. | Fleet overlay subnet spans AZ1 and AZ2. Identity Broker (VIDB) migrated from VM Management VLAN to fleet overlay. |
| DD-MGT-012 | Fleet Manager | VCF Operations Fleet Manager deployed on fleet overlay segment with dedicated IP. | All downstream fleet firewall rules reference the Fleet Manager overlay IP. Documented in Firewall Rules Workbook. |

The diagram on the following page (Figure X) illustrates the complete NSX architecture for this deployment, showing all layers from the Federation control plane through to the distributed firewall enforcement points.